A

PROJECT SEMINAR REPORT

on

PREDICTIVE MAINTENANCE OF AIRCRAFT ENGINES

Submitted in partial fulfilment for the Award of Degree of BACHELOR OF ENGINEERING

IN

INFORMATION TECHNOLOGY

BY

M.Sumedhaa (160118737019)

&

G.Usha (160118737021)

Under the guidance of

Mr.S.Rakesh

Asst.Professor

DEPARTMENT OF INFORMATION TECHNOLOGY CHAITANYA BHARATHI INSTITUTE OF TECHNOLOGY (A) (Affiliated to Osmania University; Accredited by NBA (AICTE) and NAAC (UGC), ISO Certified 9001:2015), Kokapet (V), GANDIPET(M), HYDERABAD – 500 075

Website: www.cbit.ac.in

2021-2022

CERTIFICATE

This is to certify that the project seminar report entitled “Predictive Maintenance of Aircraft engines” submitted to Chaitanya Bharathi Institute of Technology, in partial fulfilment of the requirements for the award of degree of B.E (Information Technology) during the academic year 2021-22 is a record of original work carried out by M.Sumedhaa (160118737019) and G.Usha (16011873721) during the period of study in the Dept. of IT, CBIT, Hyderabad.

Project Guide

S.Rakesh

Asst Professor, IT Dept. CBIT, Hyderabad.

Head of the Department Dr. K. Radhika.

Professor, IT Dept

CBIT, Hyderabad.

ii

ACKNOWLEDGEMENT

We would like to express our heartfelt gratitude to S.Rakesh, our project guide, for her invaluable guidance and constant support, along with her capable instruction and persistent encouragement.

We are grateful to our Head of Department, Dr.K.Radhika, for his steady support and the provision of every resource required for the completion of this project.

We would like to take this opportunity to thank our Principal, Dr.P.Ravinder Reddy, as well as the management, for having designed an excellent learning atmosphere.

Our thanks to all members of the staff and our lab assistants for helping us to carry out the groundwork of this project.

We also take this opportunity to thank our parents for their support to complete the project.

iii

ABSTRACT

In the past if a mechanic was servicing a machine and found unusual visual or acoustic behaviour in a certain part, the machine may be shut down before breaking and the part was exchanged.That is already predictive maintenance. The new thing today is that machines and algorithms can perform this task 24/7 based on smart algorithms, more data and automated alerts for detecting irregularities.In predictive maintenance scenarios, data is collected over time to monitor the state of equipment.As in most ML projects, there is a need for enough historical data to understand previous failures.

Applications of predictive maintenance are massive cost savings mainly due to decreased downtime,avoidance of related machinery failure due to connected parts,reduction of storage cost for spare parts.Predictive maintenance determines the condition of equipment and predicts when maintenance should be performed.The goal is to find patterns that can help to predict and ultimately prevent failures.

Failure prediction is a major topic in predictive maintenance in many industries. Aircrafts are particularly interested in predicting equipment failures in advance so that they can enhance their operations and reduce delays.The aim of predictive maintenance in aviation is first to predict when a component failure might occur, and secondly, to prevent the occurrence of the failure by performing maintenance..Observing engine's health and condition through sensors and telemetry data is assumed to facilitate this type of maintenance by using machine learning algorithms.

iv

TABLE OF CONTENTS

Title Page No Abstract iv

List of figures

List of tables

1. INTRODUCTION

vi vii

1.1 Overview 1 1.2 Applications 1

1

1.3 Problem Definition

1.4 Aim of the Project

2.LITERATURE SURVEY

2

2.1 PAPER 1 2 2.2 PAPER 2 3 2.3 PAPER 3 5 2.4 PAPER 4 7

3.SYSTEM REQUIREMENT SPECIFICATION

3.1 Software Requirements 9 3.2 Hardware Requirements 9

4.PROPOSED SYSTEM /METHODOLOGY

4.1 System Architecture/ System Design/ Algorithms 10

5. CONCLUSION 12 BIBLIOGRAPHY 13

v

LIST OF FIGURES

Name of Figure Page no.

Fig 2.1.5 Paper 1 Diagram 3 Fig 2.2.5 Paper 2 flowchart 4 Fig 2.3.5 Paper 3 flowchart 6 Fig 2.4.5 Paper 4 flowchart 8 Fig 4.1: Flow chart of the project. 10

vi

LIST OF TABLES

Name of Table Page no.

Table 2.1.7 Paper 1 Metrics table Table 2.2.7 Paper 2 Metrics table Table 2.3.7 Paper 3 Metrics table Table 2.4.7 Paper 4 Metrics table

3

4

7

9

vii

1.INTRODUCTION

1.1 Overview

In the past if a mechanic was servicing a machine and found unusual visual or acoustic behaviour in a certain part, the machine may be shut down before breaking and the part was exchanged. That is already predictive maintenance. The new thing today is that machines and algorithms can perform this task 24/7 based on smart algorithms, more data and automated alerts for detecting irregularities.

Predictive maintenance is challenging to implement but the potential benefits outperform the efforts by far. While there are various reasons why an actual deployment in many cases takes more time and effort than expected.In predictive maintenance scenarios, data is collected over time to monitor the state of equipment.As in most ML projects, there is a need for enough historical data to understand previous failures.

Moreover, general static features of the system can also provide valuable information, such as mechanical properties, average usage and operating conditions.The goal is to find patterns that can help to predict and ultimately prevent failures. 1.2 Applications

● Massive cost savings mainly due to decreased downtime.

● Avoidance of related machinery failure due to connected parts. ● Reduction of storage cost for spare parts.

● The maintenance team can focus on long term improvements rather than emergency repairs.

● A precious treasure of data will be generated that can be used for all kinds of other optimising tasks.

1.3 Problem Definition

Failure prediction is a major topic in predictive maintenance in many industries. Aircrafts are particularly interested in predicting equipment failures in advance so that

1

they can enhance their operations and reduce delays.Observing the engine's health and condition through sensors and telemetry data is assumed to facilitate this type of maintenance by using machine learning algorithms.

1.4 Project Aim

Predictive maintenance determines the condition of equipment and predicts when maintenance should be performed.In predictive maintenance scenarios, data is collected over time to monitor the state of equipment. The goal is to find patterns that can help to predict and ultimately prevent failures.The aim of predictive maintenance in aviation is first to predict when a component failure might occur, and secondly, to prevent the occurrence of the failure by performing maintenance.

2. LITERATURE SURVEY

2.1 PAPER 1

2.1.1 TITLE: Predictive Maintenance in Aviation: Failure Prediction from Post Flight Reports

2.1.2 AUTHOR : Panagiotis Korvesis,Stephane Besseau and Michalis Vazirgiannis 2.1.3 SITE & YEAR OF PUBLICATION: IEEE Transactions on Industrial Informatics,2018

2.1.4 DESCRIPTION :

In this paper an approach to tackle the problem of event prediction for the purpose of performing predictive maintenance in aviation is described. Given a collection of recorded events that correspond to equipment failures,a method to predict the next occurrence of one or more events of interest is implemented (target events or critical failures).Objective of the project is to develop an alerting system that would notify aviation engineers well in advance for upcoming aircraft failures, providing enough time to prepare the corresponding maintenance actions.

2

A regression problem is proposed in order to approximate the risk of occurrence of a target event, given the past occurrences of other events.

2.1.5 DIAGRAM :

Fig 2.1.5 Paper 1 Diagram

2.1.6 PROS & CONS:

Pros: A multiple instance learning scheme (multiple instance regression) is proposed based on event logs.

Cons: Used event log data only,information from a variety of sources, such as sensors,maintenance logs and environmental variables are not used.

2.1.7 METRICS:

Table 2.1.7 Paper 1 Metrics table

| Method | Precision | Recall F1 score |
| --- | --- | --- |
| SVM | 0.21 | 0.02 0.03 |
| RFR | 0.64 | 0.23 0.34 |

2.2 PAPER 2

2.2.1 TITLE: Predictive Maintenance of Aircraft Engine using Deep Learning Technique

2.2.2 AUTHOR:AdePitra Hermawan, Dong-Seong Kim and Jae-Min Lee 2.2.3 SITE & YEAR OF PUBLICATION: IEEE Transactions on Industrial Informatics,2020

3

2.2.4 DESCRIPTION :

In this paper, an accurate algorithm to estimate the remaining useful life of an aircraft engine is proposed. Since the aircraft engine has a low fault tolerance, meaning that a little fault in the system can lead to catastrophic conditions, accurate and real-time information about the engine condition is required. This paper utilizes the combination of CNN and LSTM algorithms in learning the behavior of the historical data and providing the accurate information about the time to failure of the system. The simulation results demonstrate that the proposed system is able to achieve improved performance in terms of accuracy rate and computing time compared to the previous works.

2.2.5 FLOWCHART :

Fig 2.2.5 Paper 2 flowchart

2.2.6 PROS & CONS :

Pros: In this paper, an algorithm to estimate remaining useful life of aircraft engines is proposed that utilizes the combination of CNN and LSTM algorithms. Cons: No complex data is considered.

2.2.7 METRICS:

Table 2.2.7 Paper 2 Metrics table

| Metrics | CNN | LSTM Proposed CLSTM |
| --- | --- | --- |
| Accuracy rate | 89.33% | 97.32% 98.06% |

4

2.3 PAPER 3

2.3.1 TITLE : Prediction of Remaining Useful Lifetime (RUL) of Turbofan Engine using Machine Learning

2.3.2 AUTHOR : Vimala Mathew, Tom Toby and Vikram Singh

2.3.3 SITE & YEAR OF PUBLICATION: IEEE Access,2017

2.3.4 DESCRIPTION:

Predictive maintenance is the method of scheduling maintenance based on the prediction about the failure time of any equipment. The prediction can be done by analyzing the data measurements from the equipment. There are different approaches available for developing a machine learning model. In this paper, a comparative study of existing set of machine learning algorithms to predict the Remaining Useful Lifetime of aircraft’s turbofan engine is done. The machine learning models were constructed based on the datasets from turbo fan engine data from the Prognostics Data Repository of NASA. Using a training set, a model was constructed and was verified with a test data set. The results obtained were compared with the actual results to calculate the accuracy and the algorithm that results in maximum accuracy is identified. The different algorithms were compared to obtain the prediction model having the closest prediction of remaining useful life cycle in terms of number of life cycles.

5

2.3.5 FLOWCHART :

Fig 2.3.5 Paper 3 flowchart

2.3.6 PROS & CONS :

Pros: By doing predictive maintenance, failures can be predicted and maintenance can be scheduled in advance. This reduces the cost and effort for doing maintenance. It increases safety of employees and reduces lost production time.

Cons:The algorithms can be tested for more real time data and always be one step ahead in predicting the maintenance requirements.

2.3.7 METRICS:

Root mean square error values

6

Table 2.3.7 Paper 3 Metrics table

2.4 PAPER 4

2.4.1 Title:Predictive Modeling of Aircraft Systems Failure Using Term Frequency-Inverse Document Frequency and Random Forest

2.4.2 Author : Weili Yan1 and Jun-Hong Zhou1

2.4.3 Site & Year Of Publication: IEEE Transactions on Aerospace and Electronic Systems,2017

2.4.4 Description : In this paper, a predictive model is proposed to predict faults with high priority in advance by exploring the historical data of aircraft maintenance systems, and preventive maintenance can be carried out based on the prediction results of the model. Prediction of faults with different priorities in this paper are formulated as a binary classification problem. First, counts of different faults that occur in past flights are used as raw data, from which term frequency-inverse document frequency is applied to do feature extraction. Next, classification of different faults is modeled by the random forest algorithm and receiver operating characteristics curve is adopted as the performance metrics. For the training dataset, the proposed method achieves true positive rate 100% and false positive rate 0.13%, while for the testing dataset, the proposed method achieves true positive rate 66.67% and false positive rate 0.13%.

7

2.4.5 Flowchart : 2.4.6 Pros & Cons :

Fig 2.4.5 Paper 4 flowchart

Pros: As compared to other methods, the proposed method achieves the highest true positive rate 100% and the lowest false positive rate 0.13%. For the testing dataset, the proposed method achieves true positive rate 66.67% and false positive rate 0.13%.

Cons:It can be observed that the true positive rate for the testing dataset needs to be improved. To achieve this goal, future work includes exploring different methods such as under-sampling.

8

2.4.7 Metrics:

Table 2.4.7 Paper 4 Metrics table

3. SYSTEM REQUIREMENTS AND SPECIFICATIONS

3.1 SOFTWARE REQUIREMENTS

● Windows 10.

● Google Colab.

● Python3.9.

3.2 HARDWARE REQUIREMENTS

● Input device (mouse / trackpad)

● 32-bit or 64-bit processor.

● Sufficient RAM to run the program.

9

4.METHODOLOGY/ PROPOSED SYSTEM

4.1 SYSTEM ARCHITECTURE

Figure 4.1: Flow chart of the project.

4.2 PROPOSED SYSTEM

The project objective is to enhance the maintenance operations by applying data science techniques and machine learning algorithms for predicting more accurate maintenance requirements.By considering the aircraft engine’s sensor values over time, the machine learning algorithm can learn the relationship between the sensor values and changes in sensor values to the historical failures in order to predict failures in the future.

10

Some of the parameters in dataset include:

1. aircraft engine run-to-failure events

2. operational settings

3. sensors measurements

● Training data file contains aircraft engines’ run-to-failure data. 20,000+ cycle records for 100 engines.

● Test data file contains aircraft engines’ operating data without failure events recorded. Remaining cycles for each engine are provided in a separate Ground truth data file.

● Ground truth data file contains the true remaining cycles for each engine in the test data.

Exploratory Data Analysis:

Feature variability, distribution, and correlation were examined to uncover underlying structure and extract important variables.

Modeling strategies used for predictive maintenance of aircraft engines are: ● Regression Modeling algorithms-Predicting Engine’s Time-To-Failure (TTF):

To predict the number of remaining cycles before engine failure.Both linear and non-linear regression models will be implemented to predict engine’s TTF. The following machine learning algorithms and their performance metrics were calculated and evaluated namely, Linear Regression, LASSO Regression, Ridge Regression, Decision Tree Regression, Polynomial Regression, and Random Forests Regression.

● Binary Classification algorithms-Engines failure in a current period:

To predict if the engine will fail within a specific cycle window or not is determined by binary classification algorithm.Binary classification algorithms will be implemented to predict which engines will fail in the current period, e.g. remaining cycles or TTF in the range 0-30 cycles.

11

Evaluated algorithms include Logistic Regression, Decision Trees, Support Vector Machines, K Nearest Neighbors, Naive Bayes, and Random Forests.

● Multiclass classification algorithms-For maintenance to be better planned:

To predict in which cycles window or period will an engine will fail to better plan the maintenance schedule multiclass classification is used..Multiclass classification algorithms were used to predict the period which an engine will fail in.Machine learning algorithms include namely Logistic Regression, Decision Trees, Support Vector Machines,Gaussian Naive Bayes, Random Forests, and Neural Networks.

5.CONCLUSION

Predictive maintenance determines the condition of equipment and predicts when maintenance should be performed.Predictive maintenance detects the anomalies and failure patterns and provides early warnings.In predictive maintenance scenarios, data is collected over time to monitor the state of equipment.Feature variability, distribution, and correlation were examined to uncover underlying structure and extract important variables.

Modeling strategies used for predictive maintenance of aircraft engines are:Regression Modeling algorithms,Binary Classification algorithms,Multiclass classification algorithms.The algorithm predicts when an engine will fail;which engine will fail in a particular time period and how better could maintenance be scheduled.So it predicts when a component failure might occur and prevents the occurrence of the failure by performing maintenance.

12

BIBLIOGRAPHY

[1] Panagiotis Korvesis,Stephane Besseau ,Michalis Vazirgiannis, “Predictive Maintenance in Aviation:Failure Prediction from Post Flight Reports”,IEEE Transactions on Industrial Informatics,2018.

[2] AdePitra Hermawan, Dong-Seong Kim and Jae-Min Lee,“Predictive Maintenance of Aircraft Engines using Deep Learning Technique”,IEEE Transactions on Industrial Informatics,2020.

[3] Vimala Mathew, Tom Toby and Vikram Singh, “Prediction of Remaining Useful Lifetime (RUL) of Turbofan Engine using Machine Learning”,IEEE Access,2017. [4] Weili Yan1 and Jun-Hong Zhou1,”Predictive Modeling of Aircraft Systems Failure Using Term Frequency-Inverse Document Frequency and Random Forest”,IEEE Transactions on Aerospace and Electronic Systems,2017. [5] Sam Heim,Jason Clemens,James E. Steck,“Predictive Maintenance on Aircraft and Applications with Digital Twin”,IEEE Access,2020.

[6] Hongsheng Yan; Hongfu Zuo; Jianqi Tang,“Predictive maintenance framework of the aircraft system based on PHM information”,IEEE Transactions on Industrial Informatics,2020.

[7] https://www.kaggle.com/maternusherold/pred-maintanance-data [8]https://towardsdatascience.com/how-to-implement-machine-learning-for-predictive -maintenance-4633cdbe4860.

[9]https://towardsdatascience.com/predictive-maintenance-machine-learning-vs-rule based-algorithms-cb48414df448.

13